- 1.    UYGULAMA / RESPONSIBILITIES for APPLICATION :

- 2.    TANIMLAR / DEFINITIONS :

- 1.   PRODUCT DEFINITION

1.1.     Manufacturer                                                    :   ARÇELİK A.Ş.

Çayırova-İstanbul/TURKEY

1.2.     Model                                                               :   B-170  SN/HC/A

1.3.     Type                                                                 :   Single Door Fridge (**)

1.4.     Standarts conformed                                        :   TS 87  /  ISO 7371  /  BSEN 153/ EN 60 335-1/ EN 60335-24

- 2.   PRODUCT OVERALL DIMENSIONS

Removable Top Trim

Stable Top Trim       With Top    Without Top	           Metal Top & Front Trim            Metal Top & Upper Hinge

2.1.    Height   (mm)     :          840                    840              818                            840				                               820

(in.)       :            33.1                   33.1            32.2                            33.1                                     32.3

2.2     Width   (mm)      :          545                    545              540                            545                                      545

(in.)        :            21.5                   21.5             21.3                           21.5                                     21.5

2.3.    Depth   (mm)       :          600                   600              600                            600                                      600

(in.)         :           23.6                   23.6             23.6                           23.6                                     23.6

- 3.   PACKAGING

3.1.    Material             :    Corrugated cardboard

3.2.    Dimensions       		     Package Type-I / Std               Package Type-I / Std 					       Package Type-II / Frog

(Single Sheet Cardboard)        (Double Sheet Cardboard)			   (Double Sheet Cardboard)

Height   (mm)    :                    885                                             895                                                  895

(in.)      :                      35.8                                            35.2                                                 35.2

Width   (mm)     :                    575                                             580                                                  580

(in.)       :                      22.6                                            22.8                                                 22.8

Depth   (mm)     :                    595                                             595                                                  665

(in.)       :                      23.4                                            23.4                                                 26.2

- 4.   WEIGHT

Removable Top Trim

Stable Top Trim       With Top    Without Top	           Metal Top & Front Trim            Metal Top & Upper Hinge

4.1.     Unpacked  (kg)  :           31.5                    31.5           30.5                            31.5                                  31

(lb)  :           69.4                    69.4           67.2                            69.4                                  68.3

  - Packed      (kg) :           33.5                     33.5           32.5                            33.5                                 33.0

(lb)  :           73.8                     73.8           71.6                            73.8                                 72.7

Note : Weights is for standart type-I packed product (Single sheet cardboard)

- 5.   VOLUME

5.1.       Total volume(gros)           (l)          :  119

(cu.ft)    :     4.1

5.2.       Total volume(net)             (l)          :  114

- (cu.ft)    :     3.9

5.3.       Fridge volume(gros)         (l)          :  105

(cu.ft)    :     3.6

5.4.       Fridge volume(net)           (l)          :  101

- (cu.ft)   :      3.5

5.5.       Freezer volume(gros)        (l)         :    14

(cu.ft)   :      0.54

5.6.       Freezer volume(net)          (l)         :    13

- (cu.ft)   :      0.5

- 6.  SHELF AREA

6.1.      Total                                   (dm²)    :  57.3

(sq.ft)    :    6.0

6.2       Shelf  area detail :

Cabinet Shelves (2 pcs	)							 (dm²)    :  2 * 12.8 = 25.6 dm²

Crisper Cover & cabinet       				 (dm²)    :  12.2 dm²

Crisper (1 pcs	)             							 (dm²)    :    6.3 dm²

Upper door shelf (1 pcs	)						 (dm²)   :    4.4 dm²

Middle door shelf (1 pcs	)						 (dm²)    :   4.4 dm²

Bottom door shelf (1 pcs	)						 (dm²)    :   4.4 dm²

- 7.  CABINET EXTERIOR

- 7.1.       Outer case

Material                                      :  Cold rolled steel sheet

Finish                                          :  Cured organic powder coating

- 7.2       Back wall

Material                                       :  Extrusion polypropylene

Finish                                          :  Corrugated and extruded

  - Insulation                                        :   Polyurethane foam

7.4.      Leveling                                          :   By two threaded adjustable  feet

7.5.      Hinges

Upper & Lower                          :  Bright zinc plated steel pressing

- 8.   CABINET INTERİOR

  - Inner liner

Material                                       :  Vacuum formed, C-Pentane resistant PS

- 8.2.     Shelf supports                                  :  Formed integral with liner

- 8.3       Shelves					                                 :  Tempered glass with front and back profile or Wire shelves (Optional)

Number of fresh food shelves      :   2

- Material for glass shelves

- Frame                                          								 :  4 mm tempered safety glass

- Front Profile                                                  :  ABS

- Back Profile                                                  :   ABS

- Finish                                                            :   Fixed for back profile with Devcon 2-ton epoxy adhesive

- Material for wire shelves

- Frame                                          								:  5 mm (0.2”) steel wire

- Filler                                                             :  2 mm (0.08”) steel wire

- Finish                                                           :  White polyethylene powder coating

- Shelf plastic trim                                          :  Injection part / ABS

8.4.       Crisper Cover                                               :  4 mm tempered safety glass

8.5.       Crisper Cover  Front Trim                            :  İnjection part / ABS

- 8.6.       Crisper

- Number of crispers                                   :  1

- Material                                                    :  Transparent PS

- Volume (NET)                              (lt)      :  13

- (cu.ft)    :   0.48

8.7.      Thermostat

- Code                                                           :  RANCO K59P3129 or GE

- Cold cut-out                               (°C )        :  -27.0 ±1.5

- Warm Cut-out                            (°C )        :  -15 ±2

- Cold cut-in                                 (°C )        :  +5.5 ±1.1

- Warm cut in                               (°C )        :  +5.9 ±1.1

- Defrost                                                       :  Automatic

- Capillary lenght                         (mm)       :  700 +35

Fill                                                             :  R-290

8.8.     Lamp                                                            :  15W, 230V screw type E14

- 8.9.     Button                                                           :  250V, 0.5A – Deka B-20 / Arcoelectric 3100

- 9.   DOOR

- 9.1       Exterior material                                      :   Cold rolled steel sheet

- 9.2       Handle                                                      :   Ear or Flash Handle

- Material                                                :   ABS

  - Door cover (Upper & Bottom)

Material                                                :  ABS

9.4      Insulation                                                  :  Polyurethane  Foam

9.5      Reversible door option                             :  Available

- 9.6.     Shelves

Number of fresh food shelves               :   3 (Optional for expencive or budget model)

Material                                                 :   HIPS for expencive models & GPPS or HIPS for budget models

Position                                                 :   Fixed for budget model & Adjustable for expencive model

Dairy shelf cover                                   :   Optional for expencive or budget model

Material                                                 :    Transparent polystyrene - GPPS

Dairy shelf                                             :   Optional for expencive or budget model

Material                                                 :   Transparent polystyrene – GPPS or HIPS & HIPS

Egg tray                                                 :   Optional for expencive or budget model

Material                                                 :   Polypropilen - PP

Bottle gripper                                         :   Optional for expencive or budget model

Material                                                  :   Polypropilen - PP

10.   REFRIGERATING SYSTEM

- 10.1      Compressor

Manufacturer and model                             :  LG ND45 NAEM

Rated voltage                                              :  220 - 240 V AC / 50 Hz

Voltage range                              (V)          :  187-264  V AC

Rated Power                               (W)          :  56 W

Refrigerating capacity         (kcal/h )          :  42

Starting current                           (Amp.)    :  2.4 A

Insulation                                                   :   F

Cooling                                                      :   Static air cooled

Run current                                (Amp.)     :  0.3

Capacitor                                                    :  4 mF  / Ducati – Epcos – Arcotronics

- 10.2.     Compressor oil

- Type                                                           :  HTS-95

- Amount                                      (cc.)         :  210 ± 15

10.3.     Refrigerant

- Type                                                           :  R600a

- Ağırlık                                        (gr.)        :   17

- (oz.)        :   1.41

- 10.4.     Dryer

- Type                                                             :  Moleculer sieve (XH-7)

- Desiccant weight                       (gr.)            :  5

- (oz.)            :  0.18

- 10.5.       Restrictor

- Material                                                      :   Copper capillary tubing

- Inner diameter              (mm)                     :   0.66

- (in.)                      :   0.026

- Outer diameter             (mm)                     :   2

- (in.)                      :   0.08

- Length                          (mm)                    :   3000

- (in.)                       :  118.1

- 10.6.       Condenser

Material                                                    :  Bundy tubing with welded steel wires

Pass number                                             :  11

- 10.7.      Suction line

Material                                                    :  Copper

Diameter                       (mm)                 :   6.35

(in)                   :   0.25

Length                         (mm)                   :   950

(in.)                   :      37.4

- 10.8        Evaporator (Fridge)

Type                                                       :   Tube on sheet

Material                                                 :   Aluminium

- 10.9        Evaporator (Freezer)

Type                                                       :   Tube on sheet

Material                                                  :   Aluminium

10.10.       Supply cord

Type                                                     :  3 core,0.75 mm² copper conductor

Insulation                                             :  PVC

Plug                                                      :  Earthed

Length                          (mm)               :  1950

(in.)                 :      76.8

11.   PERFORMANCE

11.1.     Classification                                                :  SN / HC  A

11.2.     Temperatures at 32°C(90°F) ambient;

Fresh food compartment ( Warmest °C)     :  0 ≤ T ≤ 5

(°F)    :   32 ≤ T ≤ 41

11.3       Power consumption   ( kWh/24h.)              :  0.452   ( 165 kWh/ year – Energy class : A )

(at 25°C ambient temperature)

11.4.      Current : 0.22 Amp.  / Power                     :  50 W

11.5.       Noise level                  (dBA)                    :  37

12.   PERFORMANCE TESTS

- 12.1.     Every unit checked for normal refrigerant charge, system leaks and restriction.

12.2.     Every refrigerator tested for low voltage start at 187 VAC.(85 % ambient of nominal voltage)

- 12.3.     Input power measured and compared to rated power.

2 .4      Every refrigerator subjected to electrical safety tests.

a)  High potential test at 1250 VAC.

b)  Earth continuity test at 10 Amp.

- Leakage current test at 1.06 Vn. (254 VAC.)

3. YÖNTEM / METHOD :

4. EKLER / APPENDIX :

5. KAYNAKÇA / REFERENCES :